# R7021E project proposal: MPC for a moving goal and a moving obstacle

Group members: Dominic Addo, Abdalrahman Nasser

# The problem

In Lab 2 we built an MPC for the TurtleBot with do-mpc. It drove to a goal and kept out of circles we fixed in the world before the solver was built. Two things in it are frozen: i. the robot is a single disc of 0.105 m, and ii. every obstacle stands still.

For this project, we want the robot to carry a load. A plank on the Burger robot is about 0.50 m by 0.20 m, with a keep-out radius of about 0.27 m. The controller would then refuse a 0.30 m gap that the robot normally fits through. We also want the obstacle to move, because a real floor has people and other machines crossing it.

So we have two questions:

1. Does describing the body as a short chain of discs rather than one let the robot use gaps that the single disc rejects?

2. And if the controller is told where an obstacle will be, instead of only where it is now, does it keep clear in cases where reacting late cannot?

# What changes in the code

Lab 2 already holds the goal as a time-varying parameter, and task 4 writes a goal position into every horizon step. That is the mechanism we extend, so we are building on something we already wrote and can explain.

- The body becomes three discs spaced along the plank instead of one. Their centres turn with the heading, so each is a short expression in x, y and theta.

- The obstacle centre becomes a time-varying parameter as well. For each tick, we write where the obstacle will be at every horizon step, exactly as the goal is written now.

- The obstacle follows a trajectory we choose, so we know its future and hand it over.

- The goal's future is not given. We fit a constant velocity to its last few observed positions and extend that across the horizon.

The keep-out constraint becomes one per disc per obstacle, written against the obstacle's predicted centre rather than fixed numbers.

# Scenarios

Each step adds one thing, so a failure points at one change.

| Scenario | What is new |
| --- | --- |
| 1. Known goal path | Square body as three discs, on a circle then a second shape |
| 2. Static obstacles | Nothing new, but the gaps get tighter |
| 3. Moving obstacle | Obstacle centre as a time-varying parameter, fed forward |
| 4. Estimated goal | Constant velocity fit to the goal's observed motion |

# Timeline

| Dates | What we do |
| --- | --- |
| 12 to 19 Oct | Plank in Gazebo, three-disc footprint, Lab 2 tasks rerun. Scenarios 1 and 2. |
| 20 to 23 Oct | Obstacle centres become time-varying parameters. Scenario 3. |
| 24 to 26 Oct | Goal estimator. Scenario 4. Collect the runs, make the plots. |
| 27 to 30 Oct | Write the report and present. |

# How we will check it works

Every scenario runs twice, once with the new information and once without. That pair is the result.

- Scenario 2: the narrowest gap the robot passes, one disc against three.

- Scenario 3: the smallest real distance to the moving obstacle, given its future against only its position now.

- Scenario 4: the distance to the moving goal, estimated against holding its last seen position.

- Solve time per tick, because three discs mean three times the keep-out constraints at 10 Hz.

Distances come from Gazebo ground truth, not from what the controller believes. We save every run to a file, so the numbers can be rechecked without rerunning the simulation.

# Hardware and lab time

None. Gazebo throughout. We need the same run repeated with the same obstacle timing and the real distance measured each time, and a lab floor cannot give us that.

# Block diagram

White boxes are given to us in previous labs or come straight from Lab 2. The colored blocks mark what is new.